Smart Pill Dispenser — Hardware & Backend Integration

MicroPython Firmware Specification, Sensor Logic Preservation & Cloud API Workflow

Project: Intelligent Medication Adherence Monitoring System (MedAdhere) Firmware Directory: updated-iot/PillDispenser Status: Implemented & Integrated with Django REST Backend Core Rule: Physical sensor & actuator logic 100% strictly preserved

1. Executive Summary & Objective

This specification documents the complete synchronization between the physical ESP32 Smart Pill Dispenser and the Django cloud backend. The previous codebase ('iot_micropython') was designed around an idealized hardware setup featuring Load Cells, OLED screens, and MP3 audio modules. In contrast, the IoT developer tested and configured the actual, physical hardware in 'updated-iot/PillDispenser'.

Objective: To bridge the exact physical hardware and GPIO pinouts from 'updated-iot' to the Django backend, while guaranteeing that the internal working logic of the sensors and actuators (stepper half-stepping, hall homing, ultrasonic pulse math, and servo lid duty cycle) remains 100% untouched and preserved.

2. Hardware Breakdown: Physical Equipment vs. Discarded Modules

The table below outlines the exact equipment physically wired and active on the device, compared with the features that were pruned from the firmware to prevent crashes and missing pin errors:

| Hardware Component | Physical Model | Configured GPIO Pins | Firmware Module |
| --- | --- | --- | --- |
| Microcontroller | ESP32 NodeMCU / DevKit | WiFi Station Mode | wifi.py |
| Real-Time Clock (RTC) | DS3231 I2C Module | SDA: 23, SCL: 22 (0x68) | rtc.py |
| Home Position Sensor | A3144 Hall Effect Sensor | GPIO 33 (Internal Pull-Up) | hall_sensor.py |
| Rotary Actuator | 28BYJ-48 Stepper + ULN2003 | IN1: 13, IN2: 14, IN3: 27, IN4: 26 | stepper.py |
| Dispenser Lid Gate | SG90 Micro Servo | GPIO 25 (50Hz PWM) | servo.py |
| Hand Detector | HC-SR04 Ultrasonic | TRIG: 19, ECHO: 18 | ultrasonic.py |

Removed Non-Physical Modules:

Load Cell: HX711 Load Cell (Weight sensor) — Discarded. Adherence is verified via hand detection rather than gram difference.

Display: SSD1306 OLED Screen (I2C display) — Discarded. System relies on cloud dashboards and physical lid cues.

Audio Module: DFPlayer Mini (UART MP3 Audio) — Discarded. Voice/audio reminders removed from firmware.

Auxiliary: Buzzer / Battery ADC — Discarded. Hardware lines removed from config to prevent phantom triggers.

3. Sensor Logic Preservation (100% Intact)

Per strict operational constraints, the core algorithms driving the physical hardware in 'updated-iot' were left unchanged:

stepper.py: Stepper Half-Stepping: Uses the exact 8-step sequence [(1,0,0,0) -> (1,1,0,0) -> ... -> (1,0,0,1)], 1024 steps per 90° compartment, with 3ms coil delay.

hall_sensor.py: Hall Homing: Runs find_home() on boot, stepping forward until Pin 33 detects active-low magnet, stops motor, sleeps 300ms, and resets compartment index to 1.

servo.py: Servo Duty Calculation: Operates at 50Hz PWM using the exact formula duty = int(1638 + (angle / 180) * (8192 - 1638)), opening at 90° and closing at 0°.

ultrasonic.py: Ultrasonic Distance Math: Sends a 10µs trigger pulse, measures echo duration with time.ticks_us(), caps timeout at 30,000µs, and computes distance = (duration * 0.0343) / 2.

dispenser.py: Dispense Hand Detection Loop: Keeps the original loop checking distance <= HAND_DISTANCE (15 cm), opens the lid, holds open while hand is present, and closes when hand is removed.

4. Backend Architecture & Cloud API Integration

The firmware interacts with the MedAdhere Django backend using lightweight HTTP calls authenticated with the X-Device-Key security header:

| Endpoint | HTTP Method | Payload / Contract | System Function |
| --- | --- | --- | --- |
| /api/v1/iot/sync/time/ | GET | Headers: X-Device-Key | Fetches server time (IST) to synchronize DS3231 RTC on boot. |
| /api/v1/iot/devices/<id>/config/ | GET | Headers: X-Device-Key | Downloads full compartment bundle (times, pills, doses) into schedules.json. |
| /api/v1/iot/events/ | POST | {event_uuid, event_type, compartment, occurred_at} | Emits live events: DOSE_STARTED, DOSE_TAKEN, DOSE_MISSED, DEVICE_BOOT. |
| /api/v1/iot/heartbeat/ | POST | {battery_level, rtc_time, stepper_status, ...} | Sends status every 5 min to keep device 'ONLINE' on Caregiver dashboard. |
| /api/v1/iot/events/batch/ | POST | {events: [array of queued events]} | Flushes offline events stored during WiFi dropouts once reconnected. |

5. End-to-End Operational Workflow

The following multi-phase sequence describes the complete lifecycle from hardware power-on to cloud verification:

Phase 1: Boot-Up & Auto-Calibration

1. The ESP32 boots up and initializes I2C bus (SDA=23, SCL=22) to verify DS3231 RTC presence.

2. WiFi connects via wifi.connect_wifi() using credentials from config.py. (If offline, proceeds with local cache).

3. Calls /api/v1/iot/sync/time/ or /config/ to set the DS3231 RTC clock to Indian Standard Time (IST).

4. Downloads doctor/caregiver prescription times from backend and persists them into local schedules.json.

5. Reports DEVICE_BOOT event and flushes any prior offline events.

6. Rotates stepper until Hall sensor detects magnet on Pin 33, locking slot 1 as HOME.

7. Sets SG90 servo to 0° (closed/locked position).

Phase 2: Real-Time Schedule Monitoring

1. System enters the main loop, reading current HH:MM from DS3231 RTC every second.

2. Compares RTC time against active compartment schedules in schedules.json.

3. In the background, emits a heartbeat every 5 minutes and auto-refreshes schedules every 30 minutes.

Phase 3: Dispensing Window Execution

1. When RTC time matches a scheduled dose (e.g. 08:00 AM for Compartment 2):

2. Emits DOSE_STARTED event to backend.

3. Stepper rotates to the target compartment (e.g., Slot 2).

4. A 3-minute dispensing window activates with the ultrasonic sensor standing by.

Phase 4: Success vs. Missed Outcome

Success Case (DOSE_TAKEN): Patient places hand near the sensor (<= 15cm). SG90 servo opens to 90°. Patient retrieves medication and removes hand. Servo closes. Firmware emits DOSE_TAKEN event. Backend decrements remaining pill count and logs 100% adherence score on caregiver dashboard.

Timeout Case (DOSE_MISSED): If no hand is detected within 3 minutes, the window closes. Servo remains locked. Firmware emits DOSE_MISSED event. Backend immediately triggers escalation (WhatsApp / SMS / Caregiver notification).

Phase 5: Offline Resilience

If home WiFi is disconnected or the router fails, the dispenser does not crash. It dispenses on time using DS3231 RTC, saves event records in event_queue.json, and flushes them to the backend immediately once WiFi recovers.

6. Deployment & Testing Guide

Follow these steps to deploy and test the integrated firmware on the physical ESP32:

WiFi Setup: Step 1: Open updated-iot/PillDispenser/config.py and set WIFI_SSID and WIFI_PASSWORD to your network.

Host IP: Step 2: Ensure BACKEND_HOST reflects the local IP of your backend host (e.g. 10.98.188.253 or 192.168.x.x).

Connection: Step 3: Connect ESP32 via USB and launch Thonny IDE or mpremote.

Upload: Step 4: Upload all files from updated-iot/PillDispenser onto the root ('/') of the ESP32 filesystem.

Run: Step 5: Execute main.py. Observe serial console for RTC check, WiFi connect, Home calibration, and live scheduling.